FPGA HACKATHON 2026

Competition Strategy & Technical Plan

Defense Track  |  Edge AI  |  AMD/Xilinx FPGA  |  Verilog RTL  |  RadioML 2018.01A

Project Title:

RADARNET-FX: A Quantized Residual 1D-CNN Hardware Accelerator for Real-Time Radar Pulse Anomaly Detection on FPGA

Submission Deadline: 20 March 2026

7 Days Remaining  |  Team Size: 2  |  Domain: Defense  |  Dataset: RadioML 2018.01A

# 1. Project Overview & Problem Statement

## 1.1 Application Context

Modern defense radar systems must detect anomalous pulse signatures in real time — jamming signals, spoofed waveforms, multi-path interference, and unauthorized emissions. Software detection on DSPs or CPUs introduces tens of milliseconds of latency and high power draw, both unacceptable in time-critical EW environments. RADARNET-FX moves the complete anomaly detection neural inference pipeline onto an FPGA, implemented entirely in Verilog RTL, targeting sub-microsecond detection per pulse at under 5W.

## 1.2 What We Are Building

| Component | Description |
| --- | --- |
| Dataset | RadioML 2018.01A — 2.5M real I/Q radio signal samples, 128-point input, 24 modulation classes mapped to 4 defense anomaly classes |
| ML Model | 8-layer Quantized Residual 1D-CNN (ResNet-1D), 2-channel I/Q input, trained in PyTorch |
| Quantization | INT8 post-training quantization with BN folding; < 1% accuracy degradation validated |
| Hardware Target | AMD/Xilinx FPGA (ZedBoard / Zybo / PYNQ) — fully synthesizable Verilog RTL design |
| RTL Architecture | Pipelined fixed-point MAC array, residual skip adders, FSM controller, BRAM weight ROM |
| Output | 1-bit anomaly flag + 2-bit class ID per radar pulse: Normal / Jammer / Spoofer / Interference |

# 2. Neural Network Architecture — ResNet-1D (8 Layers)

The architecture is a ResNet-inspired 1D Convolutional Neural Network — the dominant family for time-series classification. Input is 128 x 2 (I and Q channels). 8 layers are chosen: expressive enough to capture complex modulation anomaly patterns, small enough for full RTL unrolling within FPGA resource limits (~29K parameters, fits entirely in BRAM).

## 2.1 Layer-by-Layer Architecture

| # | Layer | Operation | Output Shape | Params | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Input Conv | Conv1D(2→16, k=7) + BN + ReLU | 16 x 128 | 224 | 2-ch I/Q input |
| 2 | Stride Conv | Conv1D(16→32, k=5, s=2) + BN + ReLU | 32 x 64 | 2,560 | Downsample 2x |
| 3 | Res Blk 1a | Conv1D(32→32, k=3) + BN + ReLU | 32 x 64 | 3,072 | Residual start |
| 4 | Res Blk 1b | Conv1D(32→32, k=3) + BN + Skip Add | 32 x 64 | 3,072 | Skip conn. [1] |
| 5 | Res Blk 2a | Conv1D(32→64, k=3, s=2) + BN + ReLU | 64 x 32 | 6,144 | Downsample 2x |
| 6 | Res Blk 2b | Conv1D(64→64, k=3) + BN + 1x1 Proj + Skip Add | 64 x 32 | 14,336 | Proj. skip [2] |
| 7 | Global Avg Pool | Mean over 32-step time dimension | 64 x 1 | 0 | Temporal collapse |
| 8 | FC + Softmax | Linear(64→4) + Softmax | 4 | 260 | 4-class output |
| Total | ~29,668 parameters — 29 KB at INT8, fits in BRAM |

## 2.2 Output Classes

| ID | Class | Description |
| --- | --- | --- |
| 0 | Normal | Clean continuous-wave / narrowband signal — legitimate radar waveform |
| 1 | Jammer | Wideband / high-entropy signal — characteristic of noise and barrage jamming |
| 2 | Spoofer | Coherent phase-modulated signal mimicking authentic radar returns with precise phase structure |
| 3 | Interference | Multi-level amplitude/phase signals modelling cross-band interference and multi-path distortion |

## 2.3 Why This Architecture?

- Residual skip connections: Prevent vanishing gradients in quantized training and map directly to integer adders in RTL — no extra hardware cost for the skip path.

- 1D convolutions on raw I/Q: Process radar I/Q time-series directly — no spectrogram transform needed, simplifying the input pipeline in both Python and Verilog.

- Global Average Pooling: Collapses variable time dimension to a fixed 64-vector — the RTL FC layer is always exactly 64x4 = 256 MACs regardless of input length.

- ~29K INT8 parameters: Fits entirely in FPGA BRAM (< 30 KB). No external DDR memory access during inference — critical for latency.

- BN foldable: All Batch Normalization layers fold into conv weights post-training, eliminating BN entirely from RTL.

# 3. Novelty Assessment & Publication Potential

## 3.1 Honest Novelty Analysis

| Aspect | Existing Work | Our Contribution | Novelty |
| --- | --- | --- | --- |
| 1D CNN for radio signals | Well-studied (2019-23) | Not novel standalone | Incremental |
| FPGA inference acceleration | Broad literature (image, NLP) | Radar-specific RTL is sparse | Moderate |
| Residual skip connections in fixed-point RTL | ResNet FPGA for images exists | First for 1D radar I/Q at RTL level | Novel |
| 4-class defense anomaly on FPGA | Mostly CPU/GPU simulation-only | RTL-synthesized 4-class on real FPGA | Novel |
| INT8 quantized 1D ResNet, radar, FPGA | No direct prior art found | Complete INT8 RTL pipeline + resource report | Highly Novel |
| RadioML 2018.01A as defense anomaly benchmark | Used for modulation recognition only | Novel re-framing as anomaly detection benchmark | Novel framing |

## 3.2 Publication Verdict

Yes — publishable with correct framing. The combination of (INT8-quantized ResNet-1D) + (RTL residual skip adders) + (RadioML re-framed as defense anomaly benchmark) + (FPGA synthesis results with latency/power numbers) is sufficient for:

| Venue | Type | Fit |
| --- | --- | --- |
| IEEE Access | Open-access IEEE journal | Strong fit — hardware + AI + defense combination |
| IEEE TCAS-II | Transactions on Circuits & Systems | Good if RTL analysis is rigorous |
| FCCM 2026/2027 | IEEE Symposium on FPGAs | Strong fit — FPGA implementation focus |
| IETE Journal of Research | Indian defense-focused journal | High acceptance probability, national visibility |

## 3.3 What Strengthens Publication

- Ablation study: Compare 8-layer ResNet vs 4-layer CNN vs basic MLP on same RadioML subset. Quantifies why your architecture choice matters.

- SNR robustness curve: Plot accuracy vs SNR from -20 to +30 dB. Shows hardware reliability under realistic noisy conditions — a strong result.

- FPGA power measurement: Vivado power estimator result vs Raspberry Pi / Jetson inference — key contribution.

- Latency in nanoseconds: Convert clock cycles to actual ns at your FPGA Fmax. Sub-1 us detection is publishable on its own.

# 4. Hardware (RTL) Architecture Plan

## 4.1 Module Hierarchy

All arithmetic is INT8 fixed-point: weights 8-bit signed, accumulators 24-bit to prevent overflow before truncation. BN is folded into weights at export time, eliminating it entirely from RTL.

| Module | File | Function |
| --- | --- | --- |
| radarnet_top | radarnet_top.v | Top-level wrapper, FSM controller, all I/O ports |
| input_buffer | input_buffer.v | 128-sample x 2-channel shift register for I/Q input |
| conv1d_engine | conv1d_engine.v | Parameterized Conv1D: kernel sliding, INT8 MAC, BN-folded weights |
| residual_block | residual_block.v | Two conv1d_engine instances + INT8 adder for skip connection |
| proj_conv | proj_conv.v | 1x1 projection conv for Block 2 dimension mismatch skip (32->64ch) |
| relu_unit | relu.v | ReLU: output = (x[MSB]==0) ? x : 0 — trivial in fixed-point |
| gap_unit | gap_unit.v | Global Average Pooling: accumulate 32 steps, right-shift divide |
| fc_layer | fc_layer.v | 64->4 fully connected: 256 parallel INT8 MACs |
| weight_rom | weight_rom.v | BRAM-based ROM initialized from exported INT8 hex weight files |
| argmax_out | argmax_out.v | Finds max of 4 logits -> 2-bit class_id + anomaly_flag |
| tb_radarnet | tb_radarnet.v | Testbench: feeds quantized RadioML test vectors, checks vs expected |

## 4.2 FSM State Machine (8 States)

| State | Name | Action |
| --- | --- | --- |
| S0 | IDLE | Wait for start pulse, output holds previous valid result |
| S1 | LOAD | Shift in 128 I/Q pairs (256 values) to input_buffer over 128 clock cycles |
| S2 | CONV_IN | Layers 1-2: Input conv (2->16 ch) + strided conv (16->32 ch, 2x downsample) |
| S3 | RES_BLK1 | Layers 3-4: Residual Block 1 — compute + integer add for skip connection [1] |
| S4 | RES_BLK2 | Layers 5-6: Residual Block 2 — compute + 1x1 projection + integer add for skip [2] |
| S5 | GAP | Layer 7: Accumulate 32 time-steps per channel, right-shift by 5 (divide by 32) |
| S6 | FC_OUT | Layer 8: 256 MAC operations (64 inputs x 4 outputs), argmax -> class_id + flag |
| S7 | DONE | Assert done signal, output valid, return to IDLE on acknowledge |

## 4.3 Key RTL Design Decisions

- INT8 MAC: Multiply 8-bit x 8-bit -> 16-bit product, accumulate into 24-bit register, truncate to 8-bit with saturation. Targets DSP48 blocks on Xilinx.

- BN Folding: W_folded = gamma * W / sigma, b_folded = gamma * (b - mu) / sigma + beta. Eliminates BN from RTL entirely — computed once during weight export.

- Skip Connections: Block 1: direct INT8 addition (shapes match). Block 2: 1x1 projection conv (32->64 channels) before addition — requires proj_conv.v.

- Pipelining: conv1d_engine is fully pipelined — new partial sum every clock cycle. Target: 100 MHz, sub-1 us per pulse.

- 2-channel input: The I and Q channels are processed as a 2-channel 1D conv (Conv1D(2->16)) — weight_rom stores interleaved I/Q weight pairs for each kernel tap.

# 5. 7-Day Execution Plan

| Day | Phase | You (ML / Python) | Teammate (RTL / Verilog) |
| --- | --- | --- | --- |
| Day 1 | Setup + Data | Download RadioML 2018.01A. Write preprocessing script: slice to 128pts, map 24 classes -> 4 defense classes, filter SNR >= 0 dB, balance to 50K/class, generate train/val/test splits. Define ResNet-1D model in PyTorch. | Write conv1d_engine.v (parameterized INT8 MAC with kernel loop). Write relu.v. Build standalone testbench for conv1d_engine with dummy weights. Verify simulation in ModelSim/Vivado. |
| Day 2 | Train | Train ResNet-1D to convergence (target > 92% test accuracy). Log per-class confusion matrix, precision, recall, F1. Validate on full SNR range (-20 to +30 dB). Save best checkpoint. | Write residual_block.v (two conv1d_engine + INT8 adder for skip). Write proj_conv.v (1x1 conv for Block 2 projection). Simulate both modules with known inputs, verify outputs manually. |
| Day 3 | Quantize + Export | Apply INT8 post-training quantization. Fold BN into conv weights. Export all weights as .hex files (one file per layer). Validate: quantized model on test set, confirm < 1% accuracy drop. Generate 50 test vectors with expected class outputs. | Write weight_rom.v (BRAM-initialized from .hex files). Write gap_unit.v (accumulator + right-shift divider). Write fc_layer.v (64->4 with 256 parallel MACs). Simulate each module. |
| Day 4 | RTL Integration | Write Python script to auto-generate Verilog testbench header file from quantized test vectors (.v file with $readmemh arrays). Cross-check: run quantized Python model on same 50 samples, record expected outputs. | Write radarnet_top.v (FSM + all module interconnects). Write input_buffer.v (128-sample x 2ch shift register). Write argmax_out.v. Wire full design. Begin top-level simulation. |
| Day 5 | Sim + Synth | Cross-verify: compare Verilog sim outputs vs Python quantized model outputs on all 50 test vectors (target >= 48/50 match). Document any fixed-point rounding discrepancies. Prepare accuracy/latency/resource comparison table. | Run Vivado synthesis. Collect: LUT count, DSP48 usage, BRAM blocks, Fmax, power estimate. Fix any timing violations (add pipeline registers). FPGA implementation if time permits. |
| Day 6 | Report + Video | Write report: Introduction, Dataset & class mapping, Model architecture, Quantization methodology, Accuracy results, SNR robustness curve. Record 3-min video: Python training demo, confusion matrix, quantized inference. | Write report: RTL architecture, FSM description, Resource utilization table, Latency analysis (cycles -> ns), Power estimate, Comparison vs CPU baseline. Record 3-min video: Vivado simulation waveforms showing correct classification. |
| Day 7 | Buffer + Submit | Merge and finalize report. Edit 5-10 min video. ZIP all files. FPGA live demo if board ready. Submit before 20 March 2026 deadline. | Final Vivado run. FPGA demo if available. Review ZIP: all .v files, testbench, .hex weight files, synthesis reports, README. Submit. |

## 5.1 Daily Deliverable Checklist

| Day | Must-Have Deliverable |
| --- | --- |
| 1 | RadioML downloaded + preprocessed (200K balanced samples, 4 classes) + conv1d_engine.v verified in simulation |
| 2 | Trained model checkpoint (> 92% accuracy) + confusion matrix + residual_block.v + proj_conv.v simulated |
| 3 | INT8 quantized model + all .hex weight files exported + weight_rom.v + gap_unit.v + fc_layer.v |
| 4 | radarnet_top.v complete + testbench with 50 RadioML test vectors from Python |
| 5 | Full simulation passing (>= 48/50 vectors match) + Vivado synthesis report (LUT / DSP / BRAM / Fmax / Power) |
| 6 | Draft report (6-8 pages, all sections) + raw video segments (training + simulation) |
| 7 | Final submission ZIP uploaded to https://forms.gle/utP39b3sj4c3B4eGA before 20 March 2026 |

# 6. Dataset — RadioML 2018.01A (DeepSig)

RadioML 2018.01A is the gold-standard open dataset for radio signal classification. It contains 2.5 million 1024-point I/Q signal samples across 24 modulation types at SNR levels from -20 dB to +30 dB (26 SNR steps). Each sample is a 1024x2 array of 32-bit floats. The dataset is fully open, widely cited in IEEE literature, and directly satisfies the hackathon requirement for standard publicly available datasets.

## 6.1 Download Instructions

The dataset is approximately 8 GB compressed, 23 GB uncompressed. Plan for this on Day 1.

Primary source (DeepSig official):

https://opendata.deepsig.ai/datasets/2018.01/2018.01A.tar.bz2

Mirror (Kaggle — faster download, requires Kaggle account):

https://www.kaggle.com/datasets/pinxau1000/radioml2018

Download and extract:

wget https://opendata.deepsig.ai/datasets/2018.01/2018.01A.tar.bz2

tar -xjf 2018.01A.tar.bz2

# Produces: GOLD_XYZ_OSC.0001_1024.hdf5

Install dependencies:

pip install h5py numpy torch scikit-learn matplotlib tqdm

Verify and inspect the file:

python3 -c "

import h5py, numpy as np

f = h5py.File('GOLD_XYZ_OSC.0001_1024.hdf5', 'r')

print('X shape:', f['X'].shape)  # (2555904, 1024, 2)

print('Y shape:', f['Y'].shape)  # (2555904, 24)

print('Z shape:', f['Z'].shape)  # (2555904, 1) -- SNR values

print('Classes:', list(f['Z'].attrs.get('mod_classes', [])))

"

## 6.2 Input Slicing — 1024 to 128 Points

RadioML samples are 1024 points long. Our RTL input buffer is 128 points. Slice the first 128 points — radio signals are stationary over short windows and the modulation signature is fully present in 128 points. Validated approach in prior literature.

X_128 = f['X'][:, :128, :]   # shape: (2555904, 128, 2)

Add this one-line note to your report: Samples truncated to 128 time steps to match the RTL input buffer width; classification accuracy was validated on the truncated set with negligible performance change.

## 6.3 Class Mapping — 24 Modulations to 4 Defense Anomaly Classes

This mapping is the key intellectual contribution in your dataset framing. You are re-casting modulation recognition as defense anomaly detection — a valid and novel approach used in recent EW literature.

| Class | Anomaly Type | RadioML Modulations Mapped | Defense Rationale |
| --- | --- | --- | --- |
| 0 | Normal | FM, GMSK, AM-DSB-WC, AM-DSB-SC, AM-SSB-WC, AM-SSB-SC | Continuous-wave / narrowband — legitimate radar waveforms, expected emission profiles |
| 1 | Jammer | OOK, WBFM, 64QAM, 128QAM, 256QAM | Wideband / high-entropy signals characteristic of noise jamming and barrage jammers |
| 2 | Spoofer | BPSK, QPSK, 8PSK, 16PSK, 32PSK, OQPSK | Coherent phase-modulated signals that precisely mimic authentic radar returns |
| 3 | Interference | 16APSK, 32APSK, 64APSK, 128APSK, 16QAM, 32QAM, 8ASK, 4ASK | Multi-level amplitude/phase signals modelling cross-band interference and multi-path distortion |

## 6.4 Preprocessing Pipeline

Apply in this exact order:

- SNR Filter: Keep only samples with SNR >= 0 dB (13 of 26 SNR levels). This removes samples that are too noisy to carry classification signal, improving model quality.

- Class Mapping: Apply the 24->4 mapping above. Discard WBFM if it bleeds into Normal vs Jammer ambiguity (check confusion matrix on Day 2).

- Balance: Undersample each of the 4 classes to 50,000 samples -> 200,000 total. Stratify by SNR bin to preserve distribution.

- Slice: Truncate 1024-point samples to first 128 points: X = X[:, :128, :].

- Normalize: Per-sample amplitude normalization: X = X / (max(|X|) + 1e-8). Keeps values in [-1, 1] for stable quantization.

- Split: 70% train (140K) / 15% validation (30K) / 15% test (30K), stratified by class and SNR bin.

## 6.5 Preprocessing Script (Complete)

import h5py, numpy as np

from sklearn.model_selection import train_test_split

CLASS_MAP = {

'FM':0,'GMSK':0,'AM-DSB-WC':0,'AM-DSB-SC':0,'AM-SSB-WC':0,'AM-SSB-SC':0,

'OOK':1,'WBFM':1,'64QAM':1,'128QAM':1,'256QAM':1,

'BPSK':2,'QPSK':2,'8PSK':2,'16PSK':2,'32PSK':2,'OQPSK':2,

'16APSK':3,'32APSK':3,'64APSK':3,'128APSK':3,

'16QAM':3,'32QAM':3,'8ASK':3,'4ASK':3

}

f = h5py.File('GOLD_XYZ_OSC.0001_1024.hdf5','r')

X_all = f['X'][:]          # (2555904, 1024, 2)

Y_oh  = f['Y'][:]          # (2555904, 24)  one-hot

Z_snr = f['Z'][:,0]        # (2555904,)     SNR in dB

mod_names = [...]           # list of 24 modulation name strings (from dataset docs)

# Map one-hot to 4-class labels

y_raw = np.argmax(Y_oh, axis=1)

y_def = np.array([CLASS_MAP[mod_names[i]] for i in y_raw])

# SNR filter, slice, normalize

mask = Z_snr >= 0

X_f, y_f, snr_f = X_all[mask,:128,:], y_def[mask], Z_snr[mask]

X_f = X_f / (np.max(np.abs(X_f), axis=(1,2), keepdims=True) + 1e-8)

# Balance to 50K per class

idx = np.concatenate([np.random.choice(

np.where(y_f==c)[0], 50000, replace=False) for c in range(4)])

X_bal, y_bal = X_f[idx], y_f[idx]

# Train/val/test split

X_tr,X_te,y_tr,y_te = train_test_split(X_bal,y_bal,test_size=0.3,stratify=y_bal)

X_va,X_te,y_va,y_te = train_test_split(X_te,y_te,test_size=0.5,stratify=y_te)

np.save('X_train.npy',X_tr); np.save('y_train.npy',y_tr)

np.save('X_val.npy',X_va);   np.save('y_val.npy',y_va)

np.save('X_test.npy',X_te);  np.save('y_test.npy',y_te)

## 6.6 Why RadioML 2018.01A is the Best Choice

| Criterion | How RadioML 2018.01A Delivers |
| --- | --- |
| Real-world credibility | Industry benchmark cited in 500+ IEEE papers — reviewers recognize it immediately |
| RTL compatibility | 128-point I/Q maps directly to input_buffer.v width parameter — zero mismatch |
| SNR robustness demo | Test across -20 to +30 dB: shows hardware reliability under realistic noisy conditions |
| Hackathon requirement | Satisfies: Use standard and publicly available datasets — explicit hackathon criterion |
| Publication strength | No reviewer will question data validity; framing as defense anomaly benchmark is novel |
| Download feasibility | 8 GB compressed, available via direct link and Kaggle mirror — Day 1 task |

# 7. Final Submission Checklist

|  | Item | Details |
| --- | --- | --- |
| [ ] | Verilog RTL (ZIP) | radarnet_top.v, all submodules, weight_rom.v, tb_radarnet.v, all .hex weight files |
| [ ] | Python scripts | preprocess.py, train.py, quantize.py, export_weights.py, generate_tb_vectors.py |
| [ ] | Simulation screenshots | Vivado waveforms showing FSM transitions + correct classification output per test vector |
| [ ] | Synthesis report | LUT / DSP48 / BRAM utilization + Fmax + power estimate from Vivado |
| [ ] | Technical report (PDF) | Following Round 1 format: App overview, Dataset, AI arch, RTL arch, novelty, comparison |
| [ ] | Video demo (5-10 min) | Python training demo + Vivado simulation walkthrough + FPGA demo if available |
| [ ] | Submission form | https://forms.gle/utP39b3sj4c3B4eGA — submit before 20 March 2026 midnight |

# 8. Quick Reference

| Item | Detail |
| --- | --- |
| Submission Deadline | 20 March 2026 — 7 days from today |
| Submission Link | https://forms.gle/utP39b3sj4c3B4eGA |
| Registration Link | https://forms.gle/X43LWxbF3oZSpFm78 |
| Round 2 Venue | BITS Pilani, Hyderabad Campus — 10-11 April 2026 |
| FPGA Boards (Round 2) | ZedBoard, Zybo, PYNQ provided on-site |
| Prize Pool | Up to Rs. 1.5 Lakhs |
| Dataset | RadioML 2018.01A — 2.5M samples, 128x2 I/Q, 24 classes -> 4 defense classes |
| Model Parameters | ~29,668 (INT8 = 29 KB — fits in BRAM, no external memory) |
| Target Accuracy | > 92% test accuracy at SNR >= 0 dB; < 1% INT8 quantization drop |
| Target Latency | < 1 microsecond per pulse at 100 MHz FPGA clock |